ECEN 454 - Lab Report

Lab Number: 3

Lab Title: Cell Characterization using Spectre

Section Number: 341

Student’s Name: Abhishek Bhattacharyya

Student’s UIN: 731002289

Date: 9/25/2024

TA: Nishank Bansal

Inverter

Transient Analysis Waveform

Parameters Used:	wp = 1.1u, wn = 0.4u

Falling Delay: 	1.505 - 1.1 = 0.405 ns

Rising Delay: 		4.521 - 4.1 = 0.421 ns

Error Percentage:	(Abs(Rising - Falling) / Min) * 100 = 3.95% < 10% ✅

Delay Table

| Capacitance (fF) | Falling Delay (ns) | Rising Delay (ns) | Error Percentage (green for <10%) |
| --- | --- | --- | --- |
| 1 | 0.02664 | 0.04348 | 63.21% |
| 8 | 0.06883 | 0.08367 | 21.56% |
| 15 | 0.09868 | 0.11105 | 12.54% |
| 22 | 0.12401 | 0.13635 | 9.95% |
| 29 | 0.14936 | 0.16137 | 8.04% |
| 36 | 0.17476 | 0.18683 | 6.91% |
| 43 | 0.20001 | 0.21269 | 6.34% |
| 50 | 0.22496 | 0.23734 | 5.50% |
| 57 | 0.22496 | 0.23734 | 5.50% |
| 64 | 0.24993 | 0.26438 | 5.78% |
| 71 | 0.27545 | 0.29086 | 5.59% |
| 78 | 0.32581 | 0.34258 | 5.15% |
| 85 | 0.35251 | 0.36 | 2.12% |
| 92 | 0.37763 | 0.39347 | 4.19% |
| 100 | 0.4014 | 0.42049 | 4.76% |
| wn = 0.4 |  | wp = 1.1 |  |

AC Sim Waveform

Sink Capacitance

| Frequency (f) | Current (I) | Sink Capacitance |
| --- | --- | --- |
| 150 | 3.70E-12 | 3.92E-15 |
| 1.80E+08 | 4.32E-06 | 3.83E-15 |
| 3.04E+08 | 7.31E-06 | 3.83E-15 |
| 3.92E+08 | 9.42E-06 | 3.83E-15 |
| 4.81E+08 | 1.16E-05 | 3.84E-15 |
| 5.75E+08 | 1.38E-05 | 3.82E-15 |
| 6.79E+08 | 1.63E-05 | 3.82E-15 |
| 7.94E+08 | 1.91E-05 | 3.83E-15 |
| 9.17E+08 | 2.21E-05 | 3.84E-15 |
| 1.00E+09 | 2.40E-05 | 3.82E-15 |
|  | Avg | 3.84E-15 |

Therefore, mean sink capacitance is 3.84 femtofarad.

NAND

Transient Analysis Waveform

Parameters Used:	wp = 0.6u, wn = 0.3u

Rising Delay: 		1.755 - 1.1 = 0.655 ns

Falling Delay: 	4.801 - 4.1 = 0.701 ns

Error Percentage:	(Abs(Rising - Falling) / Min) * 100 = 7.02% < 10% ✅

Delay Table

| Capacitance (fF) | out1 | out2 | Falling Delay (ns) | Rising Delay (ns) | Error Percentage (green for <10%) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1.136 | 4.158 | 0.036 | 0.058 | 61.11% |
| 8 | 1.194 | 4.213 | 0.094 | 0.113 | 20.21% |
| 15 | 1.237 | 4.257 | 0.137 | 0.157 | 14.60% |
| 22 | 1.28 | 4.303 | 0.18 | 0.203 | 12.78% |
| 29 | 1.322 | 4.347 | 0.222 | 0.247 | 11.26% |
| 36 | 1.366 | 4.394 | 0.266 | 0.294 | 10.53% |
| 43 | 1.408 | 4.439 | 0.308 | 0.339 | 10.06% |
| 50 | 1.451 | 4.482 | 0.351 | 0.382 | 8.83% |
| 57 | 1.496 | 4.527 | 0.396 | 0.427 | 7.83% |
| 64 | 1.537 | 4.573 | 0.437 | 0.473 | 8.24% |
| 71 | 1.579 | 4.619 | 0.479 | 0.519 | 8.35% |
| 78 | 1.621 | 4.661 | 0.521 | 0.561 | 7.68% |
| 85 | 1.664 | 4.706 | 0.564 | 0.606 | 7.45% |
| 92 | 1.707 | 4.749 | 0.607 | 0.649 | 6.92% |
| 99 | 1.749 | 4.795 | 0.649 | 0.695 | 7.09% |
| w_n = 0.3 |  |  |  | w_p = 0.6 |  |

Waveform

AC Sim Waveform

Sink Capacitance

| Frequency (f) | Current (I) | Sink Capacitance |
| --- | --- | --- |
| 1.00E+08 | 1.41E-06 | 2.24E-15 |
| 2.00E+08 | 2.81E-06 | 2.24E-15 |
| 3.00E+08 | 4.21E-06 | 2.23E-15 |
| 4.00E+08 | 5.62E-06 | 2.24E-15 |
| 5.00E+08 | 7.02E-06 | 2.23E-15 |
| 6.00E+08 | 8.43E-06 | 2.24E-15 |
| 7.00E+08 | 9.83E-06 | 2.24E-15 |
| 8.00E+08 | 1.12E-05 | 2.24E-15 |
| 9.00E+08 | 1.26E-05 | 2.23E-15 |
| 1.00E+09 | 1.40E-05 | 2.24E-15 |
|  | Avg | 2.23E-15 |

XOR

Transient Analysis Waveform

Parameters Used:	wp = 1.6u, wn = 0.4u

Rising Delay: 		1.663 - 1.1 = 0.563 ns

Falling Delay: 	4.636 - 4.1 = 0.536 ns

Error Percentage:	(Abs(Rising - Falling) / Min) * 100 = 5.04% < 10% ✅

Delay Table

| Capacitance (fF) | out1 | out2 | Falling Delay (ns) | Rising Delay (ns) | Error Percentage (green for <10%) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1.146 | 4.152 | 0.046 | 0.052 | 13.04% |
| 8 | 1.188 | 4.193 | 0.088 | 0.093 | 5.68% |
| 15 | 1.222 | 4.224 | 0.122 | 0.124 | 1.64% |
| 22 | 1.259 | 4.258 | 0.159 | 0.158 | 0.63% |
| 29 | 1.296 | 4.292 | 0.196 | 0.192 | 2.08% |
| 36 | 1.333 | 4.326 | 0.233 | 0.226 | 3.10% |
| 43 | 1.369 | 4.361 | 0.269 | 0.261 | 3.07% |
| 50 | 1.406 | 4.395 | 0.306 | 0.295 | 3.73% |
| 57 | 1.442 | 4.43 | 0.342 | 0.33 | 3.64% |
| 64 | 1.478 | 4.462 | 0.378 | 0.362 | 4.42% |
| 71 | 1.514 | 4.496 | 0.414 | 0.396 | 4.55% |
| 78 | 1.551 | 4.532 | 0.451 | 0.432 | 4.40% |
| 85 | 1.586 | 4.565 | 0.486 | 0.465 | 4.52% |
| 92 | 1.622 | 4.598 | 0.522 | 0.498 | 4.82% |
| 99 | 1.658 | 4.632 | 0.558 | 0.532 | 4.89% |
| w_n = 0.4 |  |  |  | w_p = 1.6 |  |

Waveform

AC Sim Waveform

Sink Capacitance

| Frequency (f) | Current (I) | Sink Capacitance |
| --- | --- | --- |
| 1.00E+08 | 4.72E-06 | 7.51E-15 |
| 2.00E+08 | 9.44E-06 | 7.51E-15 |
| 3.00E+08 | 1.42E-05 | 7.51E-15 |
| 4.00E+08 | 1.89E-05 | 7.51E-15 |
| 5.00E+08 | 2.36E-05 | 7.51E-15 |
| 6.00E+08 | 2.83E-05 | 7.50E-15 |
| 7.00E+08 | 3.30E-05 | 7.50E-15 |
| 8.00E+08 | 3.77E-05 | 7.50E-15 |
| 9.00E+08 | 4.24E-05 | 7.49E-15 |
| 1.00E+09 | 4.71E-05 | 7.49E-15 |
|  | Avg | 7.51E-15 |

Summary of Parameters

|  | WP | WN |
| --- | --- | --- |
| Inverter | 1.1u | 0.4u |
| NAND | 0.6u | 0.3u |
| XOR | 1.6u | 0.4u |

Files

Cell18.spi

inverter.spi

Nand2.spi

Xor.spi

simcap_inverter.spi

simcap_nand2.spi

simcap_xor.spi