# ECEN 454 - Lab Report

Lab Number: 4

Lab Title: Design & Simulation of 1-bit Adder

Section Number: 341

Student’s Name: Abhishek Bhattacharyya

Student’s UIN: 731002289

Date: 10/20/2024

TA: Nishank Bansal

# Introduction

During this lab, we faced issues with the LVS licenses for Assura. For this reason, I was unable to create any extracted views for testing in post-layout simulation. Therefore, all numbers below are from the schematics of the respective part. I did my best to carefully check each layout to manually verify correctness since layout-checking tools were not available.

For each gate, I provided two designs and sets of data. The first design is optimized for device size and power efficiency (I set WN = 300nm, the smallest setting, and attempted to get below 10% error from there).  This was done to provide a baseline for delay. I then tried to improve the delay by optimizing the design to minimize delay. I limited myself to a maximum device size of 2 um, (2000nm), as anything above this is excessive. I also tried to make the delay for inverter, NAND2, and XOR be as similar as possible so that each gate performs similarly within the 1-bit adder. I used the designs with the minimum delay for the 1-bit adder.

# Inverter (Baseline)

## Schematic

### Sizing Used: WP = 850n, WN = 300n

## Waveform

## Rising/Falling Delay

### Falling Delay: 	3.30435 - 3.05 = 0.25435 ns

### Rising Delay: 		27.4272 - 27.15 = 0.2772 ns

### Error Percentage:	(Abs(Rising - Falling) / Min) * 100 = 8.98% < 10% ✅

## Power Dissipation

### VDD: 3.476 µW

### V_Pulse: 19.43 nW

# Inverter (Improving Design to Minimize Delay)

## Schematic:

### Sizing Used: WP = 1.1µ, WN = 400n

## Waveform

## Rising/Falling Delay

### Falling Delay: 	3.2674 - 3.05 = 0.2174 ns

### Rising Delay: 		27.3731 - 27.15 = 0.2231 ns

### Error Percentage:	(Abs(Rising - Falling) / Min) * 100 = 2.62% < 10% ✅

## Power Dissipation

### VDD: 3.541 µW

### V_Pulse: 31.42 nW

# NAND2 (Baseline)

## Schematic:

### Sizing Used: WP = 300n, WN = 300n

## Waveform:

## Rising/Falling Delay:

### Falling Delay: 	72.4935 - 72.15 = 0.3435 ns

### Rising Delay: 		96.3886 - 96.05 = 0.3386 ns

### Error Percentage:	(Abs(Rising - Falling) / Min) * 100 = 1.45% < 10% ✅

## Power Dissipation

### VDD:  2.080 µW

### V_Pulse (A): 8.374 nW

### V_Pulse (B): 9.471 nW

# NAND2 (Improving Design to Minimize Delay)

## Schematic:

### Sizing Used: WP = 900n, WN = 1µ

## Waveform

## Rising/Falling Delay

### Falling Delay: 	72.3142 - 72.15 = 0.1642 ns

### Rising Delay: 		96.2087 - 96.05 = 0.1587 ns

### Error Percentage:	(Abs(Rising - Falling) / Min) * 100 = 3.47% < 10% ✅

## Power Dissipation

### VDD: 2.307 µW

### V_Pulse (A): 27.07 nW

### V_Pulse (B): 30.62 nW

# XOR (Baseline):

## Schematic:

### Sizing Used: WP = 1.1µ, WN = 300n.

### Also used the baseline sizing for the inverter: WP = 850n, WN = 300n

## Waveform:

## Rising/Falling Delay

### Falling Delay: 	27.6151 - 27.15 = 0.49429 ns

### Rising Delay: 		3.54429 - 3.05 = 0.4651 ns

## Error Percentage:	(Abs(Rising - Falling) / Min) * 100 = 6.28% < 10% ✅

## Power Dissipation

### VDD: 3.329 µW

### V_Pulse (A): 44.96 nW

### V_Pulse (B): 55.55 nW

# XOR (Improving Design to Minimize Delay):

## Schematic:

### Sizing Used: WP = 2µ, WN = 550n.

### I also used the improved sizing for the inverter: WP = 1.1µ, WN = 450n

## Waveform:

## Rising/Falling Delay

### Falling Delay: 	27.5377 - 27.15 = 0.3877 ns

### Rising Delay: 		3.40293 - 3.05 = 0.35293 ns

## Error Percentage:	(Abs(Rising - Falling) / Min) * 100 = 9.85% < 10% ✅

## Power Dissipation

### VDD: 4.204 µW

### V_Pulse (A): 70.73 nW

### V_Pulse (B): 87.39 nW

# 1-bit Adder

## Schematic:

## Layout:

### Sizing Used:

### Inverter:	WP = 1.1µ, WN = 400n

NAND2:	WP = 900n, WN = 1µ

XOR:		WP = 2µ, WN = 550n.

## Waveform

## Rising/Falling Delay

### Falling Delay: 	195.15 - 195.5352 = 0.3561 ns

### Rising Delay: 		171.4061 - 171.05 = 0.3852 ns

## Error Percentage:	(Abs(Rising - Falling) / Min) * 100 = 8.17% < 10% ✅

## Power Dissipation

## Max Frequency

### Testing with 1ns, the output is inconsistent:

### Testing with 2ns, the output is consistent and reaches 1.8V for every transition:

### Therefore, max frequency = 1/2ns = 500 MHz