# ECEN 454 - Lab Report

Lab Number: 9

Lab Title: Optimization using Logical Effort

Section Number: 341

Student’s Name: Abhishek Bhattacharyya

Student’s UIN: 731002289

Date: 11/17/2024

TA: Nishank Bansal

# Circuit Schematic w/ Critical Path Highlighted

# Logical Effort Calculations (G, B, H) & Procedure

## Explanation of Procedure

I used Google Sheets for all logical effort calculations, following the procedure as laid out in the lab manual and the provided Sutherland_Ch1.pdf. I have shared this google sheet at this link in case there are any questions about my calculations. My procedure and formulas were:

I first looked up the values of sink capacitances from Lab 3. This gave me:

CNAND = 2.23fF

CXOR = 7.51 fF

I then calculated the Path Logical Effort (G) by taking the product of each of the gate’s g:

G = 4 * (4/3)8 = 39.955

I then calculated Path Electrical Effort (H) as Cload / CXOR:

H = Cload / CXOR = 30 fF / 7.51fF = 3.995

Finally, I calculated Path Branching Effort (B) as the product of b0 * b1 * … * b9. I calculated bi for each device using the formulas given in the lab manual:

b1 = (CXOR + CNAND)/CXOR =  1.30

b3, 5, 7, 9 = 1

b2, 4, 6, 8 = (CXOR + CNAND)/CNAND =  4.37

B = b1 * b2 * … * b9 = 1.30 * 1 * (4.37)4 = 471.99

Finally, I calculated Path Effort (F) as F = G*B*H, and Stage Effort (f̂) as f̂ =F1/9

## Final Values

| Cnand | 2.23E-15 |
| --- | --- |
| Cxor | 7.51E-15 |
| Gxor | 4 |
| Gnand | 1.333333333 |
| G | 39.95488493 |
| H | 3.99 |
| B | 471.9928233 |
| F | 75333.23146 |
| f̂ | 3.482470127 |

# Resizing Procedure & Final Transistor Sizes

Using the procedure above, I found a value of f̂ = 3.482. Since If 2.7 < f̂ < 4, I performed the following resizing:

- For the i-th gate (i = 9, 8, … , 1), calculate Cin,i = giCout,i/ f̂

- If Cin, i < Cgate keep the size. Otherwise, resize the width of Wn and Wp by ratio Cin, i / Cgate

To do this, I first calculated Cin, i for each gate starting from i = 9 (final NAND gate, as labeled in my critical path schematic) and ending at i = 1 (first XOR gate). The gates at i =7, 5, 3, and 1 are highlighted as they have an additional capacitance for the XOR gate that needs to be added. This is reflected in the formulas given below:

| Gate (i) | Formula | Cin | Resize by (if green) |
| --- | --- | --- | --- |
| 9 (NAND) | Cin, 9 =(4/3 * Cload) /f̂ | 1.15E-14 | 5.150717473 |
| 8 (NAND) | Cin, 8 =(4/3 * Cin, 9) /f̂ | 4.40E-15 | 1.972055193 |
| 7 (NAND) | Cin, 7 =[4/3 * (Cin, 8+CXOR)] /f̂ | 4.56E-15 | 2.044437043 |
| 6 (NAND) | Cin, 6 =(4/3 * Cin, 7) /f̂ | 1.75E-15 | 0.7827536083 |
| 5 (NAND) | Cin, 5 =[4/3 * (Cin, 6+CXOR)] /f̂ | 3.54E-15 | 1.589089147 |
| 4 (NAND) | Cin, 4 =(4/3 * Cin, 5) /f̂ | 1.36E-15 | 0.6084145598 |
| 3 (NAND) | Cin, 3 =[4/3 * (Cin, 4+CXOR)] /f̂ | 3.39E-15 | 1.522339956 |
| 2 (NAND) | Cin, 2 =(4/3 * Cin, 3) /f̂ | 1.30E-15 | 0.5828582971 |
| 1 (XOR) | Cin, 1 =[4 * (Cin, 2+CXOR)] /f̂ | 1.01E-14 | 1.347402728 |

I then calculated the resizing factor for each gate as Cin, i / Cgate in the “Resize by” column. If the value is less than one, then we get Cin, i < Cgate and keep the size. I used conditional formatting to highlight all gates with a value greater than one and only these were resized.

Below are the sizing used for unoptimized and optimized gates:

## Unoptimized Gate Sizing:

| Unoptimized |  |  |
| --- | --- | --- |
| Inv | W_p | 1100n |
|  | W_n | 400n |
| NAND | W_p | 600n |
|  | W_n | 300n |
| XOR | W_p | 1600n |
|  | W_n | 400n |

## Optimized Gate Sizing:

Note: The gate numbers given here correspond to the gates on the critical path schematic (using the i variable). For the “optimized” sizing, these specific sizes were used for each gate on the critical path. For all gates NOT on the critical path, the sizing was left unchanged.

| Optimized |  |  |
| --- | --- | --- |
| NAND9 | W_p | 3090.430484 |
|  | W_n | 1545.215242 |
| NAND8 | W_p | 1183.233116 |
|  | W_n | 591.6165578 |
| NAND7 | W_p | 1226.662226 |
|  | W_n | 613.331113 |
| NAND6 | W_p | 600 |
|  | W_n | 300 |
| NAND5 | W_p | 953.4534882 |
|  | W_n | 476.7267441 |
| NAND4 | W_p | 600 |
|  | W_n | 300 |
| NAND3 | W_p | 913.4039735 |
|  | W_n | 456.7019867 |
| NAND2 | W_p | 600 |
|  | W_n | 300 |
| XOR1 | W_p | 2155.844364 |
|  | W_n | 538.9610911 |
| XOR1 (INV) | W_p | 1482.143001 |
|  | W_n | 538.9610911 |

# Simulation Waveforms

## Case 1: A=0000, B=1111, Carry In=1

### Unoptimized

### Optimized

## Case 2: A=1010, B=0101, Carry In=0

### Unoptimized

### Optimized

## Case 3: A=1010, B=0101, Carry In=1

### Unoptimized

### Optimized

## Case 4: A=1100, B=1000, Carry In=0

### Unoptimized

### Optimized

# Delays, Unoptimized vs. Optimized

Note: I used conditional formatting here to color the cells and make any improvement in delay for the optimized design easier to spot. Delay is evaluated as Rise Time - 3.05, which was the measured time of the rising edge of the input signals (shown in the simulation waveforms).

|  |  | Rise Time (ns) |  | Delay (ps) |  |
| --- | --- | --- | --- | --- | --- |
| Case | Pin | Non-Optimized | Optimized | Non-Optimized | Optimized |
| A=0000 B=1111 Carry In=1 | CARRY | 4.08236 | 3.8496 | 1032.36 | 799.6 |
|  | SUM<3> | 4.00451 | 3.96193 | 954.51 | 911.93 |
|  | SUM<2> | 3.80472 | 3.76949 | 754.72 | 719.49 |
|  | SUM<1> | 3.60535 | 3.58622 | 555.35 | 536.22 |
|  | SUM<0> | 3.414 | 3.4089 | 364 | 358.9 |
| A=1010 B=0101 Carry In=0 | CARRY | 4.07565 | 3.91008 | 1025.65 | 860.08 |
|  | SUM<3> | 4.11396 | 4.11813 | 1063.96 | 1068.13 |
|  | SUM<2> | 3.91069 | 3.90091 | 860.69 | 850.91 |
|  | SUM<1> | 3.70815 | 3.70387 | 658.15 | 653.87 |
|  | SUM<0> | 3.51061 | 3.51071 | 460.61 | 460.71 |
| A=1010 B=0101 Carry In=1 | CARRY | 4.08243 | 3.84992 | 1032.43 | 799.92 |
|  | SUM<3> | 4.00465 | 3.96219 | 954.65 | 912.19 |
|  | SUM<2> | 3.8049 | 3.76974 | 754.9 | 719.74 |
|  | SUM<1> | 3.60555 | 3.58656 | 555.55 | 536.56 |
|  | SUM<0> | 3.414 | 3.40889 | 364 | 358.89 |
| A=1100 B=1000 Carry In=0 | CARRY | 3.50037 | 3.3417 | 450.37 | 291.7 |
|  | SUM<3> | 3.82718 | 3.82914 | 777.18 | 779.14 |
|  | SUM<2> | 3.69247 | 3.67924 | 642.47 | 629.24 |
|  | SUM<1> | 3.62019 | 3.61334 | 570.19 | 563.34 |
|  | SUM<0> | 3.45202 | 3.45214 | 402.02 | 402.14 |

# Power Consumption, Unoptimized vs. Optimized

The optimized design consumed ~10 uW more power for all tests. This is expected, as the larger transistor sizing causes higher power consumption.

|  | Power |  |
| --- | --- | --- |
| Case | Non-Optimized | Optimized |
| A=0000 B=1111 Carry In=1 | 55.73 uW | 63.74 uW |
| A=1010 B=0101 Carry In=0 | 105.5 uW | 115.4 uW |
| A=1010 B=0101 Carry In=1 | 55.89 uW | 63.89 uW |
| A=1100 B=1000 Carry In=0 | 81.59 uW | 93.68 uW |

# Area, Unoptimized vs. Optimized

Here, the area is in units of picometers squared. Calculations are given in the Appendix below.

| Area |  |
| --- | --- |
| Non-Optimized | Optimized |
| 20.19 pm2 | 71.6 pm2 |

# Appendix: Area Calculations

I calculated the area by finding the area of each transistor type (XORNMOS, XORPMOS, etc.) and then multiplying this by the product of the number of transistors in each gate and the number of instances of each gate in the design. For the unoptimized design, I considered each of the critical path gates (i = 9, 8, …, 1) separately. I used the below table of formulas for the calculations.

| Unoptimized |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Gate | Transistor | Width | Length | # of Gates | # Transistors (each) | Area |
| Inv | W_p | 1.10E-06 | 2.00E-07 | 16 | 1 | 3.52E-12 |
|  | W_n | 4.00E-07 | 2.00E-07 | 16 | 1 | 1.28E-12 |
| NAND | W_p | 6.00E-07 | 2.00E-07 | 12 | 2 | 2.88E-12 |
|  | W_n | 3.00E-07 | 2.00E-07 | 12 | 2 | 1.44E-12 |
| XOR | W_p | 1.60E-06 | 2.00E-07 | 8 | 4 | 1.02E-11 |
|  | W_n | 4.00E-07 | 2.00E-07 | 8 | 4 | 2.56E-12 |
|  |  |  |  |  |  | 2.19E-11 |
| Optimized |  |  |  |  |  |  |
| Gate | Transistor | Width | Length | # of Gates | # Transistors (each) | Area |
| Inv | W_p | 1.10E-06 | 2.00E-07 | 14 | 1 | 3.08E-12 |
|  | W_n | 4.00E-07 | 2.00E-07 | 14 | 1 | 1.12E-12 |
| NAND | W_p | 6.00E-07 | 2.00E-07 | 4 | 2 | 9.60E-13 |
|  | W_n | 3.00E-07 | 2.00E-07 | 4 | 2 | 4.80E-13 |
| XOR | W_p | 1.60E-06 | 2.00E-07 | 7 | 4 | 8.96E-12 |
|  | W_n | 4.00E-07 | 2.00E-07 | 7 | 4 | 2.24E-12 |
| NAND9 | W_p | 3.09E-06 | 2.00E-07 | 1 | 2 | 1.24E-12 |
|  | W_n | 1.55E-06 | 2.00E-07 | 1 | 2 | 6.18E-13 |
| NAND8 | W_p | 1.18E-06 | 2.00E-07 | 1 | 2 | 4.73E-13 |
|  | W_n | 5.92E-07 | 2.00E-07 | 1 | 2 | 2.37E-13 |
| NAND7 | W_p | 1.23E-06 | 2.00E-07 | 1 | 2 | 4.91E-13 |
|  | W_n | 6.13E-07 | 2.00E-07 | 1 | 2 | 2.45E-13 |
| NAND6 | W_p | 6.00E-07 | 2.00E-07 | 1 | 2 | 2.40E-13 |
|  | W_n | 3.00E-07 | 2.00E-07 | 1 | 2 | 1.20E-13 |
| NAND5 | W_p | 9.53E-07 | 2.00E-07 | 1 | 2 | 3.81E-13 |
|  | W_n | 4.77E-07 | 2.00E-07 | 1 | 2 | 1.91E-13 |
| NAND4 | W_p | 6.00E-07 | 2.00E-07 | 1 | 2 | 2.40E-13 |
|  | W_n | 3.00E-07 | 2.00E-07 | 1 | 2 | 1.20E-13 |
| NAND3 | W_p | 9.13E-07 | 2.00E-07 | 1 | 2 | 3.65E-13 |
|  | W_n | 4.57E-07 | 2.00E-07 | 1 | 2 | 1.83E-13 |
| NAND2 | W_p | 6.00E-07 | 2.00E-07 | 1 | 2 | 2.40E-13 |
|  | W_n | 3.00E-07 | 2.00E-07 | 1 | 2 | 1.20E-13 |
| XOR1 | W_p | 2.16E-06 | 2.00E-07 | 1 | 4 | 1.72E-12 |
|  | W_n | 5.39E-07 | 2.00E-07 | 1 | 4 | 4.31E-13 |
| XOR1 (INV) | W_p | 1.48E-06 | 2.00E-07 | 2 | 4 | 2.37E-12 |
|  | W_n | 5.39E-07 | 2.00E-07 | 2 | 4 | 8.62E-13 |
|  |  |  |  |  |  | 7.16E-11 |